| CONTACT SUPPORT: |  |
| --- | --- |
| COMPANY NAME: | CSEED Entertainment Systems |
| SUPPORT CONTACT: | Alexander Kampichler |
| EMAIL ADDRESS: | ak@cseed.tv |
| PHONE: | +43 664 4648105 |
| ADDRESS: | Frankenberggasse 13 / Vienna / Austria |
| NOTES: |  |

| GENERAL INFORMATION |  |
| --- | --- |
| SIMPLWINDOWS NAME: | CSeed_Backend_TCPDriver |
| CATEGORY: | TV Control |
| VERSION: | V1.0 |
| SUMMARY: | Controlling of TV movement/brightness/volume; Indication of actual status |
| GENERAL NOTES: |  |
| CRESTRON HARDWARE REQUIRED: | CP4N – Serial No. 2142JBH01089 |
| SETUP OF CRESTRON HARDWARE: | CP4N with CrestronGo |
| VENDOR FIRMWARE: | 2.4508.00032 |
| VENDOR SETUP: |  |
| CABLE DIAGRAM: | One TCP-IP connection to the Backend Control PC |

| CONTROL: |  |  |
| --- | --- | --- |
| Signal/Function Name | D,S,A | Digital, Serial, Analog signal property definition. |
| diBackendActive | D | Set to “1” for activation of the backend for TV control |
| diAcActive | D | Set to “1” for activation of AC (Audiocontroller) control |
| diExtend | D | Press to extend the TV |
| diRetract | D | Press to retract the TV |
| diStop | D | Press to stop the movement of the TV |
| diTurnLeft | D | Press to turn the TV left |
| diTurnRight | D | Pulse to turn the TV right |
| diMoveUp | D | Press to move the TV up (just vertical TVs) |
| diMoveDown | D | Press to move the TV down (just vertical TVs) |
| diBrightnessUp | D | Press to increase the TV brightness |
| diBrightnessDown | D | Press to decrease the TV brightness |
| diVideoInSat | D | Press to change the video input on the Denon amplifier for the TV to SAT (HDMI) |
| diVideoInDvd | D | Press to change the video input on the Denon amplifier for the TV to DVD (HDMI) |
| diVideoInBd | D | Press to change the video input on the Denon amplifier for the TV to Blu-Ray (HDMI) |
| diVideoInGame | D | Press to change the video input on the Denon amplifier for the TV to Game (HDMI) |
| diVideoInMPlay | D | Press to change the video input on the Denon amplifier for the TV to Mediaplayer (HDMI) |
| diVideoInAux | D | Press to change the video input on the Denon amplifier for the TV to AUX (HDMI) |
| diVideoInCd | D | Press to change the video input on the Denon amplifier for the TV to CD (HDMI) |
| diVolumeUp | D | Press to increase the TV volume |
| diVolumeDown | D | Pulse to decrease the TV volume |
| diVolumeMuteOn | D | Pulse to mute the TV volume |
| diVolumeMuteOff | D | Press to unmute the TV volume |
| diAcExtend | D | Press to extend the speakers |
| diAcRetract | D | Press to retract the speakers |
| diAcStop | D | Press to stop the speaker movement |
| diAcVolumeMuteOn | D | Pulse to mute the speakers volume |
| diAcVolumeMuteOff | D | Press to unmute the speakers volume |
| diAcGroup1 | D | Press to limit control to speaker group 1 |
| diAcGroup2 | D | Press to limit control to speaker group 2 |
| diAcGroup3 | D | Press to limit control to speaker group 3 |
| aiBrightnVal | A | Use to set the brightness of the TV |
| aiVolumeVal | A | Use to set the volume of the TV |
| aiAcVolumeVal | A | Use to set the volume of the speakers |
| aiAcColorR | A | Use to set the red color amount of the speaker LEDs |
| aiAcColorG | A | Use to set the green color amount of the speaker LEDs |
| aiAcColorB | A | Use to set the blue color amount of the speaker LEDs |
| siIpAddress | S | Use to set the IP address of the backend PC or Audiocontroller (depending on the inputs “diBackendActive” and “diAcActive”) |

| FEEDBACK: (*examples below) |  |  |
| --- | --- | --- |
| doInfoHatchAopen | D | Indicates when hatch A of the TV is opening in automatic mode |
| doInfoHatchAclose | D | Indicates when hatch A of the TV is closing in automatic mode |
| doInfoHatchBCopen | D | Indicates when hatch B/C of the TV is opening in automatic mode |
| doInfoHatchBCclose | D | Indicates when hatch B/C of the TV is closing in automatic mode |
| doInfoTvColLift | D | Indicates when the TV column is lifting in automatic mode |
| doInfoTvColRetr | D | Indicates when the TV column is retracting in automatic mode |
| doInfoTvColTurn | D | Indicates when the TV column is turning in automatic mode |
| doInfoTvUnfold | D | Indicates when the TV is unfolding in automatic mode |
| doInfoTvFold | D | Indicates when the TV is folding in automatic mode |
| doInfoSecPinUnl | D | Indicates when the secure pins of the TV are unlocking in automatic mode |
| doInfoSecPinLo | D | Indicates when the secure pins of the TV are locking in automatic mode |
| doInfoConfLo | D | Indicates when the confirmation button of the TV is locked in automatic mode |
| doInfoConfUnl | D | Indicates when the confirmation button of the TV is unlocked in automatic mode |
| doInfoTvStop | D | Indicates when the TV movement was stopped in automatic mode |
| doInfoTvDepl | D | Indicates when the TV is completely deployed in automatic mode |
| doInfoTvRetr | D | Indicates when the TV is completely retreated in automatic mode |
| doInfoConfNotPr | D | Indicates when a movement button was pressed via the appropriate input, but the confirmation button on the TV is not pressed |
| doInfoKeySwNotRem | D | Indicates when a movement button was pressed via the appropriate input, but the key switch on the TV is in the wrong position |
| aoBrightnVal | A | Indicates the actual brightness value of the TV |
| aoVolumeVal | A | Indicates the actual volume value of the TV |
| aoAcVolumeVal | A | Indicates the actual volume value of the speakers |
| aoAcColorR | A | Indicates the actual red amount of the speakers LEDs |
| aoAcColorG | A | Indicates the actual green amount of the speakers LEDs |
| aoAcColorB | A | Indicates the actual blue amount of the speakers LEDs |
| soAcGroup1 | S | Indicates the name of speaker group 1 (get’s read from the Audiocontroller as soon as a communication via input “diAcActive” and proper set IP address is established) |
| soAcGroup2 | S | Indicates the name of speaker group 2 (get’s read from the Audiocontroller as soon as a communication via input “diAcActive” and proper set IP address is established) |
| soAcGroup3 | S | Indicates the name of speaker group 2 (get’s read from the Audiocontroller as soon as a communication via input “diAcActive” and proper set IP address is established) |

| TESTING: (please fill out carefully) |  |
| --- | --- |
| OPS USED FOR TESTING: | Windows 10 Pro Version: 21H2 |
| SIMPL WINDOWS USED FOR TESTING: | Version 4.18p1 |
| DEVICE DB USED FOR TESTING: | 200.150 |
| CRES DB USED FOR TESTING: | 210.05 |
| SYMBOL LIBRARY USED FOR TESTING: | Symlib2.Tio 1159 |
| SAMPLE PROGRAM: | (Name of the program code specific for each processor used) – don’t know what exactly is meant |
| REVISION HISTORY: | (Version of the module being released) – don’t know what exactly is meant |