| CONTACT SUPPORT: |  |
| --- | --- |
| COMPANY NAME: | Gude Systems GmbH, Germany |
| SUPPORT CONTACT: | Hornung Engineering, Germany |
| EMAIL ADDRESS: | info@hornung-engineering.com |
| NOTES: |  |

| GENERAL INFORMATION |  |
| --- | --- |
| SIMPLWINDOWS NAME: | Gude Module 8316 v6.0.umc |
| CATEGORY: | Power Controller |
| VERSION: | 6.0 |
| SUMMARY: | The module controls the PDU and IP power strip and relay module of GUDE Expert Power Control 8316 through TCP/IP. It can switch relays on/off and meter different power quantities. |
| GENERAL NOTES: | Http/Https password protection is supported. |
| CRESTRON HARDWARE REQUIRED: | 3-Series processor, 4-Series processor |
| SETUP OF CRESTRON HARDWARE: | The demo program was written for a CP3. The GUDE Expert Power Control is controlled via TCP/IP. Port: 80 (HTTP) or 443 (HTTPS) |
| VENDOR FIRMWARE: | V1.3.3 |
| VENDOR SETUP: | Expert Power Control 8316 |
| CABLE DIAGRAM: | Standard ethernet cable. |

| CONTROL: |  |  |
| --- | --- | --- |
| Signal/Function Name |  |  |
| Enable_HTTPS | D | Set the input to "1" to enable HTTPS. |
| Px_on | D | Press to switch the port x on. |
| Px_off | D | Press to switch the port x off. |
| Px_reset | D | Press to reset the port x. |

| FEEDBACK: |  |  |
| --- | --- | --- |
| Px_on_fb | D | Indicates the port x is on. |
| Px_off_fb | D | Indicates the port x is off. |
| Px_name | S | Indicates the current port name. |
| Px_voltage | S | Indicates the voltage at the port x in V. |
| Px_current | S | Indicates the current consumption of port x in A. |
| Px_power | S | Indicates the power consumption of port x in W. |
| Px_icurrent | A | Indicates the current consumption of port x in mA. |
| Sx_temperature | S | Indicates the temperature measured by the external sensor x. |
| Sx_humidity | S | Indicates the humidity measured by the external sensor x. |
| Sx_pressure | S | Indicates the pressure measured by the external sensor x. |
| Line_Voltage | S | Indicates the voltage at the line-port in V. |

| PARAMETERS: |  |  |
| --- | --- | --- |
| ipadress | S | IP address of the device being controlled. |
| username | S | The username for the password protection (if enabled). |
| password | S | The password for the password protection (if enabled). |

| TESTING: |  |
| --- | --- |
| OPS USED FOR TESTING: | CP3N 1.601.3935.27221 |
| SIMPL WINDOWS USED FOR TESTING: | V 4.11.06 |
| DEVICE DB USED FOR TESTING: | V 200.00.015.00 |
| CRES DB USED FOR TESTING: | V 200.00.004.00 |
| SAMPLE PROGRAM: | Gude Demo 8316 v6.0.smw |
| REVISION HISTORY: | V 6.0 Bugfixes |